XRipper XRP

Manufacturer:	Vogelsang

Model:		XRP __________ Q

Twin-shaft shredder for shredding coarse and solid materials in a liquid stream.

Vertical, space-saving design with drive located directly above the shredding chamber. Compact design with low footprint for pipeline installation.

Efficient protection against clogging and blockages in pipes, pumps, and fittings

Economical processing of solids in liquid media

Suitable for various coarse and solid materials

Design and operating data:

Conveyed medium

Throughput 		______m³/h

Solid content		______

Temperature 		__________°C

pH value 		___________

Operating pressure             	_____________

Vogelsang XRipper XRP

Seal as cartridge unit

Robust monolithic (one-piece) rotors

Ripper rotors can be removed without dismantling the piping

Materials and design

Ripper rotors		Highly wear-resistant special steel

Cutting blade width	_____ mm

Rotor design		monolithic

Liquid-contacted

Housing parts		______________

Protective plates		Highly wear-resistant special steel

Shaft seal	Mechanical seal, medium-cooled

with sealing chamber and

leakage indicator

Mechanical seal pair	Block ring/Duronit with NBR O-ring

In contact with liquid

O-rings		NBR

Connections

XRipper Connections with parking feet

Material		_____________________

Inlet			Compatible with DN _____ EN1092-1/11, PN ___

Outlet			Compatible with DN _____ EN1092-1/11, PN ___

Inlet side

Connection box  		 yes ___  no ___

Drive

Flat gear motor

Manufacturer 		Getriebebau Nord

Type 			________________

Power  		___________ kW

Output speed 	___________ rpm

Voltage 		400 V

Frequency 		50 Hz

Protection class 		IP 55

ISO class 		F

Temperature sensor 	3

The XRipper and motor are connected directly to each other via a steel torque support (painted).

Paint finish:

XRipper	:	Primer steel protection, top coat

RAL3020, traffic red

Drive:	Primer steel protection, top coat

RAL3020, traffic red

Total weight		_________________________

Dimensions (LxWxH)     _________________________

XRipper as described above

Options:

Control unit mounted in control cabinet

Control cabinet XRipper XRipper

Local control panel

Dimensions (H x W x D): 500 mm x 500 mm x 210 mm

Material: Steel

XRipper power: 2.2 kW

XRipper load circuit: Direct

Includes software / blockage management

Controller with installed software

Pre-assembled on mounting rail for installation

in a control cabinet

Control module

Type: XRipper PCU

Data communication: ProfiNet

Remote control interface: Via data communication

Supply voltage: 24V DC

Protection class: IP20

Dimensions (W x H x D): 95 x 128 x 46 mm

Mounting type: DIN rail

Includes software / blockage management