XRipper Giant

Type: XRG __________ QD HCD / HCS

Manufacturer:			Vogelsang

Type:			XRG ________ QD

Compact, space-saving twin-shaft grinder for grinding coarse and solid materials in sewer installations

Efficient protection against clogging and blockages in pipes, pumps, and fittings

Economical treatment of solids in liquid media

Suitable for various coarse and solid materials

Self-cleaning flow expansion designed as a rotating stainless steel screen drum

Design and operating data:

Conveyed medium

Throughput 		______m³/h

Solid content		______

Temperature 		______°C

pH value 		______

ATEX version:   yes  / no

- Zone: 1

- Temperature class: T4

Materials and design

Ripper rotors		Highly wear-resistant special steel

Rotor design		Monolithic

Cutting blade width	____ mm

Liquid-contacting

housing parts    		Steel

Protective plates           	Highly wear-resistant special steel

Shaft seal 	Mechanical seal cooled by the medium,

with seal chamber and

leakage indicator

Seal carrier 	C 45

Mechanical seal pair 	Block ring/Duronit with NBR O-ring

Fluid-contacting 	O-rings  NBR

Selected materials / design:

Rotor design monolithic (one-piece)       yes    no

Cutting blade width        _________mm

Protective plates                    _________________________________

Sliding ring pairing               __________________________________

Housing parts                    ___________________________________

Drive shredding unit:

Flat gear motor in submersible design

Manufacturer 		Getriebebau Nord / EMOD

Type 			________________

Power  		__________ kW

Output speed 		___________ rpm

Voltage 		___________ V

Frequency 		50 Hz

Protection class 		IP 55

ISO class 		F

Temperature sensor 	3

Flow expansion with drive:

Self-cleaning screen drum in segmented design

Rotating screen drum: 		1 piece (HCS) / 2 pieces (HCD)

Ball passage: 			Ø 20 mm

Diameter:			Ø400 mm

Material: 			Stainless steel

Drive power:			0.25 kW

Motor protection class:		IP68

Speed:			14 rpm

Safety instructions:

Access to the X-Ripper must be prevented during operation.

The X-Ripper must not be started during maintenance work.

Wall mounting: straight

The XRipper SIK includes a stainless steel wall bracket, allowing it to be mounted directly in front of a channel outlet. A wall bracket with guide rail allows the entire XRipper to be lifted out of the channel using the lifting bracket for service and maintenance. When the shredder is put back into use, it is lowered back into the console via the rails. This holds it in position automatically without the need for further securing.

Includes stainless steel guide rail

Length of guide rail: ___________ mm

Design Channel shape Open channel

Channel width:  _____________ mm

Channel height:   _____________ mm

Paint finish:

XRipper	:	+++KTL primer steel protection++++, top coat

RAL3020, traffic red

Drive:		Primer steel protection, top coat

RAL3020, traffic red

Total weight		_________________________

Dimensions (LxWxH)     _________________________

XRipper as described above

Controller with installed software (for ATEX XRC and non-ATEX XRC)

Pre-assembled on mounting rail for installation

in a control cabinet

Control module

Type: XRipper PCU

Data communication: ProfiNet

Remote control interface: Via data communication

Supply voltage: 24V DC

Protection class: IP20

Dimensions (W x H x D): 95 x 128 x 46 mm

Mounting type: DIN rail

Includes software / blockage management