XRipper XRC__________QD

Manufacturer:			Vogelsang

Model:			XRC________QD

Compact, space-saving twin-shaft shredder for shredding coarse and solid materials in sewer installations

Efficient protection against clogging and blockages in pipes, pumps, and fittings

Economical treatment of solids in liquid media

Suitable for various coarse and solid materials

Design with sewer integration kit for shaft or sewer installation

Design and operating data:

Conveyed medium

Throughput 		______m³/h

Solid content		______

Temperature 		______°C

pH value 		______

ATEX version:   yes  / no

- Zone: 1

- Temperature class: T4

Vogelsang XRipper XRC________QD

Dry-running resistant

Seal as cartridge unit

Materials and design

Ripper rotors		Highly wear-resistant special steel

Cutting blade width	___ mm

Rotor design:		monolithic

Liquid-contact

housing parts    		Steel

Protective plates           	Highly wear-resistant special steel

Shaft seal 	Mechanical seal, medium-cooled,

with seal chamber and

leakage indicator

Mechanical seal pairing 	Block ring/Duronit with NBR O-ring

Fluid-contacting 	O-rings  NBR

Selected materials/design:

Rotor design monolithic (one-piece)       yes    no

Cutting blade width        _________mm

Protective plates                    _________________________________

Sliding ring pairing             _________________________________

Housing parts                    _________________________________

Drive

Flat gear motor in submersible design

Manufacturer 		Getriebebau Nord / EMOD

Type 			________________

Power  		__________ kW

Output speed 	___________ rpm

Voltage 		___________ V

Frequency 		50 Hz

Protection class 		IP 55

ISO class 		F

Temperature sensor 	3

The XRipper and motor are connected directly to each other via a steel torque support (painted).

Wall mounting: straight / round

The XRipper SIK includes a stainless steel wall bracket, allowing it to be mounted directly in front of a channel outlet. A wall bracket with guide rail allows the entire XRipper to be lifted out of the channel using the lifting bracket for service and maintenance. When the shredder is put back into use, it is lowered back into the console via the rails. This holds it in position automatically without the need for further securing.

Includes stainless steel guide rail

Length of guide rail: ___________ mm

Shaft diameter:  _____________ mm

Design Channel shape Open channel

Channel width:  _____________ mm

Channel height:   _____________ mm

Paint finish:

XRipper	:	+++KTL primer steel protection++++, top coat

RAL3020, traffic red

Drive:		Primer steel protection, top coat

RAL3020, traffic red

Total weight		_________________________

Dimensions (LxWxH)     _________________________

XRipper as described above

Options:

Control unit mounted in control cabinet (not for ATEX XRC)

Control cabinet XRipper XRipper

Local control panel

Dimensions (H x W x D): 500 mm x 500 mm x 210 mm

Material: Steel

XRipper power: 2.2 kW

XRipper load circuit: Direct

Includes software / blockage management

Controller with installed software (for ATEX XRC and non-ATEX XRC)

Pre-assembled on mounting rail for installation

in a control cabinet

Control module

Type: XRipper PCU

Data communication: ProfiNet

Remote control interface: Via data communication

Supply voltage: 24V DC

Protection class: IP20

Dimensions (W x H x D): 95 x 128 x 46 mm

Mounting type: DIN rail

Includes software / blockage management