Vogelsang rotary lobe pump

as a horizontally mounted, dry-running rotary piston pump.

The pump and drive are mounted on a shared, torsion-resistant console made of galvanized steel, aligned, and connected with an elastic coupling including a plastic coupling guard.

Process conditions:

Pumped medium:		________________________

Required volume flow (control range from – to, if applicable):____________________

TS content:		_________________________

Medium temperature:	_________________________

Pressure at inlet:	_________________________

Pressure at outlet:	_________________________

Differential pressure:		_________________________

Design and materials:

Make:		 Vogelsang

Model:			VX136 Q

Selected manufacturer:	___________________

Selected type:		___________________

Pump speed:	___________________

Power requirement:	___________________

Max. ball passage: 40 mm

Shaft diameter of pump chamber: 60 mm

Pump:

Housing half-shells:		Adjustable housing halves made of GG-25

Axial housing protection:		Protective plates in HVSS

Rotary pistons:			four-wing HiFlo pistons in pulsation-free geometry made of NBR

Bearing:			Single-sided bearings for quick piston replacement

Shaft:				robust and break-proof shaft

Shaft seal:		Seal as cartridge unit

Material: Block ring 1.4301 Cr2O3 / Duronit

Seal inspection:		Visual inspection of the mechanical seal using a pressurized seal chamber container

Corrosion protection:		2-component primer and top coat

Pump and motor in RAL3020 (traffic red)

Maintenance:			QuickService concept

Connections:

Connection type:		___________________

Connection standard:	EN1092-1/11 PN16

Connection size:	___________________

Material:		___________________

Drive:

Helical gear motor

Manufacturer:		Getriebebau Nord

Type:			_________________________

Drive power:	_________________________

Output speed:		_________________________

ISO class:			F

Protection class:			IP55

Voltage:			400/690 V

Frequency:			50 Hz

Winding protection:		3 PTC thermistors

Control range during frequency converter operation: 	___________________________

Supply the pump system described above.

Options:

InjectionSystem

Housing half-shells with an increased wrap angle of at least 190°

Advantages:

- Improved efficiency

- Improved suction behavior

- Longer piston service life

- Greater resistance to foreign objects

Radial protective plates

Pump housing with modular injection system and radial protection plates. Maintenance possible without removing the pump from the pipeline.

Dry-running protection

The pump is supplied with a pre-installed temperature sensor for monitoring the temperature of the pump.

The scope of delivery includes a 5 m cable and a suitable evaluation device with display for front panel installation. The evaluation device is supplied separately.

Make PT100:		___________________________

Type PT100:		___________________________

Fabrikat Temperaturregler:________________________

Temperature controller type:	___________________________

Digital pressure monitoring

Maximum pressure shut-off and pressure sensor for control

2x digital output, 1x analog output

3-digit LED