RotaCut wet shredder

RotaCut for reliable and economical shredding of fibers and impurities and separation of heavy materials.

Operating conditions:

Medium				__________

Throughput capacity:			______m³/h

Viscosity:				Good flow

Temperature:				______°C

Solid content:				______

Particle length after comminution: 	______ mm

Density:					______ kg/m³

pH value:				______

Operating pressure:				______ bar

- Shredder with integrated heavy material separator

- Easy and quick access to the cutting unit and heavy material separator

- Mechanical seal without flushing and barrier water connection

- Asymmetrical knife rotor

Materials and design

Cutting head

Housing material:	___________

Self-sharpening cutting blades made of hardened stainless steel

Cutting screen made of highly wear-resistant special steel

Duronit/Duronit sliding ring pairing with NBR O-ring

O-rings in contact with medium NBR

Separator with connections

Material:		___________

Suction nozzle 		Compatible with DN _______ EN1092-1/11, PN __

Pressure connection 		compatible with DN _______ EN1092-1/11, PN __

Cleaning opening 	___________  Square flange

Drive

Flat gear motor

Make:

Type:

Drive power: 		_______ kW

Output speed:		_______

ISO class:			F

Protection class:			IP55

Voltage:			400 V

Frequency:			50 Hz

Winding protection:		3 PTC thermistors

Automatic cutting force control (ACCplus)

Automatic cutting force control using compressed air for infinitely variable adjustment of the contact pressure of the cutting blades on the cutting screen during operation.

Shutdown of the system when the blades reach their wear limit.

No deflection of the cutting blades possible, thus ensuring optimum cutting performance with low preload pressure and low wear

Adjustment of the contact pressure by means of a compressed air connection or manual application

Sensor for registering the wear status of the cutting blades.

Paint finish:

Primer steel protection, top coat RAL3020 (traffic red)

Dimensions (LxWxH):	mm

Accessories:

RotaCut control module

consisting of

Hardware

Data communication: 	__ (Profinet)

__ (Local I/O)

Supply voltage: 24V DC

Protection class: IP20

Temperature range: 0°C to 55°C

Mounting type: DIN rail

CE conformity: Certificate available

cULus compliance: Certificate available

Software

Functions:

Visualization

Automatic operation

Manual operation

Remote automatic mode

Web-enabled visualization based on HTML5

Motor thermistor, motor protection switch, and load circuit evaluation

Blockage management

Operating data acquisition (e.g., operating hours, etc.)

Overload and underload detection

Evaluation of pressure monitoring on the suction and pressure sides

RotaCut RCQ control cabinet

Dimensions (H x W x D): 500 mm x 500 mm x 210 mm

Material: Steel

Protection class IP55

Functions

Visualization

Automatic operation

Manual operation

Motor thermistor, motor protection switch, and load circuit evaluation

Blockage management

Overload detection

Evaluation of pressure monitoring on suction and pressure side

Evaluation of ACC pressure switch

Evaluation of cutting blade wear condition